# 002LED_Tasks – Real-Time LED Control with Deterministic FreeRTOS Scheduling

## Overview

This project demonstrates deterministic periodic task execution, preemptive multitasking, and real-time scheduling behavior using FreeRTOS on the STM32 NUCLEO-F401RE (ARM Cortex-M4) platform.

Three independent RTOS tasks are created, each responsible for controlling a different LED with a unique execution period. The project highlights the correct professional approach for implementing periodic real-time tasks, and clearly demonstrates the differences between:

- ❌ Blocking delay (HAL_Delay)

- ⚠️ Non-deterministic RTOS delay (vTaskDelay)

- ✅ Deterministic periodic delay (vTaskDelayUntil)

This implementation reflects real-world RTOS design patterns used in embedded firmware, industrial control systems, robotics, and automotive ECUs.

## Hardware Platform

Development Board: STM32 NUCLEO-F401RE
Microcontroller: STM32F401RET6 – ARM Cortex-M4 (84 MHz)
RTOS Kernel: FreeRTOS
Development Environment: STM32CubeIDE
Debug Interface: SEGGER SystemView via UART/RTT

## Task Architecture

Three independent FreeRTOS tasks are created using xTaskCreate():

| Task Name | Hardware Resource | Execution Period | Function |
| --- | --- | --- | --- |
| Green LED Task | GPIO PC0 | 800 ms | Periodic LED control |
| Orange LED Task | GPIO PC1 | 400 ms | Periodic LED control |
| Red LED Task | GPIO PC2 | 200 ms | Periodic LED control |

Each task runs as an independent execution entity with:

- Its own stack

- Its own execution context

- Equal priority level

- Independent scheduling lifecycle

This demonstrates true RTOS-based multitasking.

## Deterministic Scheduling using vTaskDelayUntil()

This project uses the correct and professional RTOS timing mechanism:

vTaskDelayUntil(&last_wakeup_time, pdMS_TO_TICKS(period));

Each task initializes its reference time using:

TickType_t last_wakeup_time = xTaskGetTickCount();

This ensures that each task executes at a precisely defined periodic interval, independent of execution overhead.

Unlike traditional delay mechanisms, this approach guarantees:

- Deterministic execution

- No timing drift

- Precise periodic scheduling

- Real-time system stability

This is the industry-standard method for periodic task implementation.

## Comparison of Delay Mechanisms

### HAL_Delay() — Not RTOS Safe

HAL_Delay(200);

Problems:

- Blocks the CPU

- Prevents scheduler operation

- Breaks multitasking

- Not suitable for RTOS systems

### vTaskDelay() — RTOS Safe but Non-Deterministic

vTaskDelay(pdMS_TO_TICKS(200));

Advantages:

- Allows scheduler operation

- Non-blocking

Disadvantage:

- Execution period includes task runtime

- Causes cumulative timing drift

### vTaskDelayUntil() — Deterministic and Correct

vTaskDelayUntil(&last_wakeup_time, pdMS_TO_TICKS(200));

Advantages:

- Precise periodic execution

- No drift

- Deterministic timing

- Professional RTOS behavior

This is the recommended approach for real-time embedded systems.

## FreeRTOS Scheduler Behavior

All tasks are created with equal priority:

Priority = 2

FreeRTOS scheduler performs:

- Preemptive scheduling

- Automatic context switching

- Efficient CPU utilization

- Task state management (Running, Ready, Blocked)

When tasks call vTaskDelayUntil(), they enter the Blocked state, allowing the scheduler to execute other ready tasks.

This ensures efficient multitasking.

## Real-Time Debugging using SEGGER SystemView

SEGGER SystemView integration enables:

- Real-time task monitoring

- Scheduler activity visualization

- Context switch tracking

- Execution timing analysis

- CPU usage observation

Debug messages are transmitted using:

SEGGER_SYSVIEW_PrintfTarget("Toggling LED");

This provides deep visibility into RTOS behavior.

## Cortex-M DWT Cycle Counter Integration

The Cortex-M Data Watchpoint and Trace unit is enabled:

#define DWT_CTRL (*(volatile uint32_t*)0xE0001000)

DWT_CTRL |= (1<<0);

This enables:

- High-precision timing

- Accurate RTOS event tracing

- Professional-level real-time debugging

## FreeRTOS APIs Used

Task Management

- xTaskCreate()

- vTaskStartScheduler()

Timing Management

- vTaskDelayUntil()

- xTaskGetTickCount()

- pdMS_TO_TICKS()

Debug and Monitoring

- SEGGER_SYSVIEW_Conf()

- SEGGER_SYSVIEW_PrintfTarget()

- SEGGER_UART_init()

## System Behavior Summary

After scheduler startup:

- All tasks execute independently

- Each LED toggles at its precise period

- Scheduler manages execution automatically

- CPU utilization is optimized

- System operates deterministically

This demonstrates professional real-time firmware design using FreeRTOS.

## Key Learning Outcomes

This project demonstrates practical understanding of:

- RTOS-based multitasking

- Deterministic task scheduling

- Periodic task implementation

- Real-time system timing control

- Scheduler-driven execution

- Embedded RTOS debugging techniques

## Conclusion

This project demonstrates the correct and professional implementation of periodic tasks using FreeRTOS.

Using vTaskDelayUntil() ensures deterministic, stable, and predictable real-time system behavior, which is essential for professional embedded firmware development.

Platform: STM32 NUCLEO-F401RE
RTOS: FreeRTOS
Debug Tool: SEGGER SystemView
Language: Embedded C